# PIKU V2 Software Architecture Documentation

Stable OOP Baseline v1

| Project | PIKU V2 - ESP32-Based Environmental Inspection and Monitoring Robot |
| --- | --- |
| Author / Engineer | Md Habibur Rahman Habib |
| Role | Developer |
| Purpose | Software architecture freeze documentation for future PIKU V2.1 development |

| Baseline | This document summarizes the current stable, hardware-tested OOP software architecture. It is intended for faculty, supervisor, and project review. |
| --- | --- |

# 1. Project Overview

PIKU V2 is a low-cost ESP32-based environmental inspection and monitoring robot. It supports manual web control, autonomous obstacle avoidance, an ST7789 TFT display, a local browser dashboard, and environmental monitoring through DHT22, MQ2, and ultrasonic sensors.

# 2. Refactor Objective

The refactor improved the internal software structure while preserving the working hardware behavior. The main goals were:

- Improve motor reliability.

- Improve modularity and separation of responsibilities.

- Make debugging easier through clearer module boundaries.

- Prepare the codebase for future PIKU V2.1 expansion.

- Preserve existing working behavior after each phase.

# 3. Layered Software Architecture Diagram

Architecture Layers [ main.cpp ] Coordinator: setup, loop, mode switching, manual safety, auto navigation v [ Application / Service Layer ] DriveCommandHandler ¦ DashboardPublisher ¦ WebDashboard v [ Hardware Driver Layer ] MotorController ¦ ServoScanner ¦ SensorManager ¦ DisplayManager v [ Configuration Layer ] PinConfig ¦ RobotConfig v [ Physical Hardware ] Motors + L298N ¦ Servo ¦ DHT22 ¦ MQ2 ¦ Ultrasonic ¦ ST7789 TFT

# 4. Module Responsibility Table

| Module / File | Layer | Responsibility | Current Status |
| --- | --- | --- | --- |
| PinConfig | Config | Central GPIO mapping for TFT, motors, servo, ultrasonic sensor, DHT, MQ2, and buzzer. | Stable |
| RobotConfig | Config | Shared tuning constants for TFT size, thresholds, scan angles, and update intervals. | Stable |
| MotorController | Hardware driver | Motor drive behavior, MotorState, pivot/spin turns, digital L298N control, and timing guards. | Stable |
| DriveCommandHandler | Application / service | Routes manual dashboard commands to motor actions and preserves command display state. | Stable |
| ServoScanner | Hardware driver | Positions the servo at center, left, right, or custom angles with attach/detach handling. | Stable |
| SensorManager | Hardware driver | Reads raw ultrasonic distance, DHT temperature/humidity, and MQ2 analog values. | Stable |
| DisplayManager | Hardware driver | Renders the ST7789 TFT dashboard, status values, and robot face. | Stable |
| DashboardPublisher | Application / service | Publishes robot state to TFT, web dashboard data model, and serial log. | Stable |
| WebDashboard | Application / service | Runs local web dashboard, mode endpoints, turn-mode endpoint, command endpoints, and JSON data output. | Stable |
| main.cpp | Coordinator | Owns setup, loop, mode switching, manual obstacle safety, sensor orchestration, scanAt, and auto navigation. | Stable coordinator |

# 5. Protected Stable Baseline Values

| Area | Protected values |
| --- | --- |
| Motor | DIRECTION_SETTLE_MS = 5 FORWARD_STARTUP_COMPENSATION_MS = 80 BACKWARD_STARTUP_COMPENSATION_MS = 0 PWM is not used currently |
| Servo scan | CENTER_SCAN = 90 LEFT_SCAN = 160 RIGHT_SCAN = 20 |
| Pins | MQ2_PIN = GPIO35 BUZZER_PIN = GPIO33 |
| Robot behavior | Manual mode is default Pivot turn and spin turn both exist Auto navigation remains in main.cpp MQ2 investigation/calibration is paused |

# 6. Manual Control Flow Diagram

Manual Command Path Web Button Press v WebDashboard v DriveCommandHandler v MotorController v L298N Motor Driver v Motors

Manual command routing was extracted into DriveCommandHandler. Manual obstacle safety intentionally remains in main.cpp so the tested safety behavior stays close to the coordinator loop.

# 7. Sensor and Dashboard Data Flow Diagram

Sensor and Dashboard Data Flow DHT22 / MQ2 / Ultrasonic v SensorManager v main.cpp v DashboardPublisher +----+----------------+ v v DisplayManager WebDashboard v v TFT Screen Browser Dashboard

SensorManager reads raw values. DashboardPublisher is responsible for publishing the latest robot state to the TFT display, browser dashboard, and serial log.

# 8. Auto Mode Flow

| Auto Navigation Flow Auto Mode in main.cpp v ServoScanner positions servo v SensorManager reads distance v main.cpp compares front / left / right distance v MotorController moves robot |  |  |
| --- | --- | --- |
| Important note | Auto navigation is intentionally still inside main.cpp because it is timing-sensitive and hardware-tested. It should not be extracted without a dedicated test plan. |  |

# 9. Do Not Break Rules

- Do not tune motor timing without hardware testing.

- Do not add PWM unless explicitly planned.

- Do not change servo scan angles casually.

- Do not change manual mode default.

- Do not modify MQ2 thresholds while MQ2 investigation is paused.

- Do not extract AutoNavigator without a dedicated test plan.

- Do not rewrite the whole project.

# 10. Remaining main.cpp Responsibilities

- Object wiring.

- Setup and loop coordination.

- Mode switching.

- Manual obstacle safety.

- Sensor read/publish coordination.

- scanAt sequence.

- Auto navigation logic.

- Gas status classification.

# 11. Future PIKU V2.1 Expansion

Future work can build on this baseline once each change has a clear test plan. Candidate improvements include:

- AutoNavigator extraction.

- Compass-based heading correction.

- Improved mapping.

- Flame sensor.

- IR sensor.

- ESP32-CAM / camera integration.

- Buzzer restoration.

- Better MQ2 calibration.

- Optional PWM motor tuning later.

# 12. Final Freeze Statement

| Freeze statement | This document freezes the current stable OOP software baseline after successful build and hardware validation. Future development should preserve this baseline unless a planned, tested refactor is performed. |
| --- | --- |